ILLINOIS TOLLWAY									AC-110

CHECKLIST FOR

EARTH EXCAVATION AND EMBANKMENT

This checklist has been prepared to provide the field inspector a summary of easy-to-read step-by-step requirements relative to the proper construction of all earthwork (Section 200).  The following questions are based on information found in Tollway Supplemental Specifications, Tollway Standards, IDOT Standard Specifications, IDOT Highway Standards, IDOT Construction Manual and current policy memorandums and letters.

Have you checked the contract Special Provisions, Supplemental Specifications and plans to see if any modifications have been made to the requirements listed herein?	____

# 1.	PRELIMINARY STAKES AND MEASUREMENTS

Prior to the moving of any earth have you:

a.	Have you cross sectioned all borrow and rock excavation sites?  (202.07 (b) & 204.07)	____

b.	Have you cross sectioned all proposed embankment sites to re-establish the ground surface?	____

c.	Are you the Contractor slope staked for both cuts and fills at 100 feet for rural and 50 feet urban intervals left and right of centerline?	____

d.	Have you determined the mimimum tree and plant material needed to be removed then:

Measured and marked for removal necessary trees (201.04)	____

Delineated and protected as needed plant material to be saved (201.05)	____

2.	EROSION CONTROL

a.	Prior to moving any earth have you:

- Reviewed the project Erosion Control and Storm Water Pollution Prevention Plan? 	____

- If applicable, reviewed the U. S. Army Corps of Engineers Permit Conditions? 	____

- Conducted a field review of all potentially erodable areas with the Engineer?	____

- Recorded locations and types of erosion control recommendations on a set of plans or by memorandum, etc.?	____

b.	Have you reviewed the “Commitment File”? (A-38)	____

c.	Are you the Contractor constructing all permanent erosion control features as soon as practical? (See Section 107.23 of the Tollway Supplemental Specifications)	____

d.	Are you ordering temporary erosion control systems to be installed to control erosion? (See Section 280 of the Tollway Supplemental Specifications)	 ____

e.	Are you the Contractor taking sufficient precautions to prevent pollution and siltation of streams, lakes, reservoirs and adjacent properties?   (See Section 107.23 of the Tollway Supplemental Specifications)	____

f.	Are you reviewing the erosion controls and ordering repairs and additional measures as necessary?  	____

- Maintenance of installed temporary erosion control systems and the installation of additional tempoary erosion control systems not provided for in the plans will be paid for in accordance with Art. 109.04 of the Tollway Supplemental Specifications. EXCEPT:

(1)	Erosion control at furnished borrow pits, storage and plant sites, and haul roads will not be paid for.	____

(2)	Erosion control required because of negligence or carelessness on the part of you the Contractor will not be paid for. 	____

# 3.	CLEARING

Are you the Contractor removing and disposing of all obstructions such as fences, walls, foundations, buildings, accumulations of rubbish, existing structures, all logs, shrubs, brush, grass, weeds, other vegetation and stumps of less diameter than 6 inches?  Clearing shall be restricted to the minimum necessary to construct the project.  (201.01(a))	____

# 4.	TREE, SHRUB, BUSH AND ROOT REMOVAL

a.	Are all trees (and stumps), bushes and roots within the slope limits of embankment 2 feet or more in depth being cut off at ground level?  (201.04 & 201.08)	____

b.	Are all other trees, stumps, saplings, bushes and roots within the right-of-way (excluding those designated to be saved) being removed to a depth of not less than 12 inches below the elevation of the subgrade, the finished earth surface, or the ground line?  (201.04 & 201.08)	____

c.	If you the Contractor elects to burn the organic waste, is it being done in accordance with the IEPA open burning permit?  Is the IEPA being notified within one working day of citizen complaints?  Is non-compliance being reported to your Supervisor?  	____

d.	Is a completed and signed Notice of Open Burn Form being provided to the IEPA not less than seven working days prior to each burn?	____

# 5.	TOP SOIL EXCAVATION

a.	Are you the Contractor obtaining top soil from only the locations and to the depths designated on the plans?  (211.03)	____

b.	Is the top soil being stock-piled at locations approved by the CM?  (211.03)

c.	Is the appriopriate erosion control system, ( i.e. temporary seeding) being applied to the topsoil stockpiles?	____

d.	Are you the Contractor required to furnish any additional top soil (1081.05) from outside the ROW, is it being approved first?  (211.03)	____

# 6.	PREPARATION OF EXISTING GROUND SURFACE

a.	After all embankment sites are cleared, is the top 6 inches of the ground surface being disked and then compacted to the satisfaction of the Engineer?  (205.03)	____

b.	If embankments are constructed during the winter, is all snow, ice and frozen earth being removed from the area to be covered by the embankment?  (205.03)	____

c.	When construction is resumed after any winter shutdown period, is the top 8 inches of all partially completed embankments reprocessed and recompacted to the minimum specified density before allowing you (the Contractor) to place more fill?  (205.03)	____

d.	Where embankment material is being placed against a hillside, sloped surface, or existing embankment, is the existing slope being plowed deeply, or benched or stepped to eliminate the possibility of a shear plane being built into the fill?  (205.03)	____

e.	Where building embankments over existing Full-Depth Bituminous, PCC Base Course or PCC Pavement, remove, break or leave existing pavement as follows:    (205.03(b))	____

# 7.	EMBANKMENT

a.	Is each layer of embankment material being leveled to not more than 8 inches, loose condition, prior to compaction?  (205.04)	____

b.	Is each layer of embankment material being placed and compacted the entire width and length of the fill area so far as practicable before beginning another lift?  (205.04)	____

c.	Is the embankment within the area of the roadbed free of sod, frost, or other material which might cause settlement if allowed to remain?  (205.04)	____

d.	Is all broken concrete 2 sf max. area, rock or stone (any size) being placed and adequately distributed in embankments?  (202.03 & 205.04)	____

# LOADS ON PIPES

Have you the Contractor been informed about which heavy equipment will need to cross over pipe or box culverts on the contract?

a.	Is earth cover over pipes sufficient to prevent damage from heavy loads?  (542.04(h))	____

b.	Is earth cover built to proper configuration to prevent damage from impact and wide enough to provide for two-way traffic?  (542.04(h))	____

# EQUIPMENT ON PAVEMENT

Have you the Contractor informed the CM what equipment will need to operate on existing pavements on the contract, and where they intend to cross?

Is any of your (Contractor’s) equipment that exceeds legal load limits prohibited from operating on pavements except at specified crossing locations or as allowed by permit?  (Art. 107.16 of the Tollway Supplemental Specifications)	____

# 10.	PLACING MATERIAL AT STRUCTURES

a.	If embankment is being placed at structures or culverts that have not been completed, is a gap of not less than 100 feet being left on each side of the structure or culvert?  (205.05)	____

b.	Is embankment that is to be placed behind completed bridge abutments or around structures not being placed until a concrete beam break of at least 650 psi or a concrete cylinder strength of at least 3500 psi is obtained and also until at least 7 days after the final concrete was placed?  (205.05)	____

c.	At closed abutments, culverts, or retaining walls, is all water and mud which causes excessive hydraulic pressures, and loose material which causes excessive earth pressures, removed before placing any backfill?  (205.05)	____

d.	In areas where the mechanical compaction equipment cannot adequately operate, is the backfill placed in 8-inch loose measurement layers and compacted with suitable mechanical hand tools?  (205.06)	____

e.	Are density tests being taken in the hand compacted areas to be sure that the required density is obtained and to make sure that the moisture content does not exceed 110% of optimum?  (205.06)	____

f.	Is a cubical deposit (2’ x 2’ x 2’) of porous course aggregate (Gradation CA 5, 7, or 11 (1004.01)) completely enclosed within a fabric envelope being deposited behind each drain hole with the backfill operations?  (205.05) & (502.10)	____

# 11.	DRAINAGE (202.05)

a.	Are you the Contractor constructing all cuts and fills in such a manner to preclude water pockets?  (202.05)	____

b.	At those locations where erosion may be a problem, are you the Contractor excavating a ditch at the toe of slope of fills and at the top of slope of cuts?  (202.05)	____

c.	Are all existing tile drains, sewers and other subsurface drains being preserved and protected?  (202.05) 	____

d.	Are you the Contractor re-directing clean water, as practicable, away from areas bare earth 	____

e.	Are you reviewing drainage areas throughout the project to ensure compliance with SWPPP and Permits?	____

# 12.	UTILITIES (Articles 105.07 & 107.31 of the Tollway Supplemental specifications)

Are you the Contractor having utilities field located and taking measures to protect the markings and facilities in an effort to avoid damage and disruption of service?  	____

# 13.	SURPLUS, UNSTABLE, AND UNSUITABLE   MATERIAL REMOVAL AND DISPOSAL

a.	Is all rubbish, and unsuitable materials being  disposed of in such a manner that public or private property will not be damaged or endangered? (202.03)	____

b.	If unsuitable material is present at or below the finished grade, is it being removed and replaced with suitable material?  (202.03)	____

c	If unstable conditions are found at grade in cut sections, is remediation of the subgrade performed in accordance with Article 301.03?	____

# 14.	COMPACTION EQUIPMENT

a.	Is your compacting equipment and compacting operations being coordinated with the rate of placing embankment so that the required density is obtained?  (205.06)	____

b.	Is a tandem disk (1101.02) being used on each layer of material?  Disking may be omitted if the fill material consists of sand, gravel, rock or broken pavement.  (205.06)	____

# 15.	COMPACTION

a.	Is the embankment being compacted to the following minimum densities? (205.06)	____

| Proposed Embankment Height | Minimum Standard Laboratory Density |  |
| --- | --- | --- |
| Less than 1.5 feet | All lifts, 95% |  |
| 1.5 feet to 3 feet |  | - First lift, 90% - Remaining lifts, 95% |
| Over 3 feet |  | - Lower 1/3 of fill (but not to exceed 2 feet, 90% - Next 1 foot of fill, 93% - Balance of fill, 95% |

b.	Are you performing job control in-place density tests within the following intervals:  (See attached schedule for sampling)	____

SAMPLING SCHEDULE 1:  EMBANKMENTS, SUBGRADES, AND GRANULAR COURSES

| MATERIAL | SPECIFICATION REFERENCE | PROPERTY/QUALITY | FREQUENCY | FORM |
| --- | --- | --- | --- | --- |
| Earth, Stone, or Gravel Embankments | Art. 205.05* | Standard Moisture Density Control Curve | Compaction curve data is required for each major change in embankment material. | No standard report. |
|  | Art. 205.05* | Density | 1 test per 20,000 cu. yd for a continuous operation, by Project Inspector. In confined areas, 1 test per 3 ft. of lift and not less than 1 test per fill area, by Project Inspector | MI 701 |
| Subgrades Subgrade Lime-modified Soil Lime (Hydrated, By-Product, Non-Hydrated) |  |  |  |  |
|  | Art. 301.03* | Density | 1 test per 1500 ft, of entire length of subgrade through both cut and fill areas, by Project Inspector | MI 701 |
|  | Art. 302.08* | Density | 1 test per 1500 ft. of treated area, by Project Inspector | MI 701 |
|  | Sec. 1012 | Various | Minimum of 1 sample on 1st day, and then 1 sample per 750 tons thereafter, by Contractor QC | LM-6 |
| Granular Courses |  |  |  |  |
| Base Course and Granular Embankment, Type A | Art. 351.05* | Density | 1 test per 1000 ft. of pavement, by CQP: Inspector | MI 701 |
| Subbase Granular Material, Type A | Art. 311.05* |  |  |  |
| Aggregate Surface Course, Type A | Art. 402.05* |  |  |  |

c.	Is the top 2 feet of all embankments at not more than 120 percent of the optimum moisture?  (You can use an approved additive at no additional cost to the contract to effect a quicker drying time.)  (205.06)	____

d.	Are you recording all density tests?	____

Note:  A low density test must be reported and cross-referenced to a retest after recompaction.

e.	Are you the Contractor correct (redisk, and/or sprinkle, and recompact) all lifts on which density is not obtained? (205.06)	____

# 16.	ROCK EXCAVATION

Is all material which is classified as rock (202.04) being excavated to a minimum depth of 3 inches below the elevation of the proposed subgrade within the limits of the roadbed, and backfilled with subbase granular material?  (202.03)  (Maximum payment for rock excavation is 6 inches below plan subgrade and 6 inches beyond neat slope lines.  202.07(b))	____

# 17.	CHANNEL EXCAVATION

a.	Are your channel excavation to the lines, grades and dimensions shown on the plans?  (203.04)	____

b.	Are you the Contractor notifying the CM at least 3 days before starting so that you can measure tree and hedge removal and take original cross sections if required?  (Art 203.04)	____

# BORROW and FURNISHED EXCAVATION

ALL requirements for BORROW EXCAVATION sites also apply to all FURNISHED EXCAVATION sites.

a.	Is the site location, excavation plan, and material approved by the engineer in writing before the Contractor starts performing the work for borrow excavation or furnished excavation until?  (Art 204.02) & (Art. 107.22 of the Tollway Supplemental Specifications)	____

b.	Tree and hedge removal furnished borrow pits are NOT measured for payment.  (Art. 201.10(b)(3))	____

c.	Are borrow pits located at least 50 ft or 10 ft plus 1 1/2 times the depth of excavation, whichever is the greater, from any highway ROW (except for points above pavement elevation) and are the slopes excavated in accordance with Article 204.02?	____

d.	Are borrow sites, any portion of which is located within 150 ft of any highway ROW submitted in accordance with Article 204.02?	____

e.	Are all borrow sites being seeded with the proper application of , fertilizer and/or gorund limestone at the rates specified?  (Art. 204.02)	____

(1)	Seeding, etc., is not required if the borrow site is to be tilled.  (The Contractor is to furnish a written statement from the owner.)  (Art. 204.02)	____

(2)	Seeding, etc., is not required below proposed water level if the borrow site is to be developed as a pond or lake.  (Art. 204.02)	____

# 19.	WASTE

Are you the Contractor being allowed to waste suitable excess excavated material off the ROW only when shown on the plans or permission in writing is given by the Engineer?  (Art. 202.03)	____

# 20.	TOP SOIL PLACEMENT

a.	Prior to top soil placement, has the area to be covered been shaped, trimmed and finished?  If the surface is eroded due to weathering, has it been filled or smoothed?  If the surface is hardened or crusted has it first been disked or raked?  (Art. 211.04)	____

b.	Is the Sub-Contractor applying the top soil to the slopes designated on the plans?  (Art. 211.04)	____

c.	Are you periodically checking the thickness of the in place top soil with hand dug holes?  (Art. 211.04)	____

# 21.	FINAL SHAPING, TRIMMING AND FINISHING

After all cuts, fills and ditches are constructed and prior to seeding, are you the Contractor performing the following operations at no additional cost to the contract:

a.	Are the ditches being cleaned and the entire roadway being shaped, trimmed and finished uniformly to the lines, grades and cross sections shown on the plans?  (Articles 212.02, 212.03 & 212.04)	____

b.	Is all unsuitable material, debris, and rubbish, and all stones or boulders more than 3 inches in largest dimension being removed from the ROW and disposed of?  (Art. 212.05)	   ____