ILLINOIS TOLLWAY									AC-120

CHECKLIST FOR

SOIL MODIFICATION

This checklist has been prepared to provide for the field inspector a summary of easy-to-read step-by-step requirements relative to the proper construction of modified soil (Section 302 of the Standard Specifications).  The following questions are based on information found in IDOT Standard Specifications, IDOT Highway Standards, IDOT Constructions Manual and current policy and memorandums and letters and Tollway Supplemental Specifications.

Have you checked the contract Special Provisions, Supplemental Specifications and plans to see if any modifications have been made to the requirements listed herein?	____

# 1.	GENERAL (Article 302.04)

Is the temperature of the soil 6 inches below the surface a minimum of 50˚ F and the ambient air temperature in the shade a minimum of 
45˚ F?	____

Is the area to be modified limited to that which could be covered with subbase, base or pavement within the same construction season?	____

# 2.	PROPORTIONING (Article 302.05)

Have you the Contractor provided samples of the soil modifier and soil to be used at least 45 days prior to the construction?	____

Has the mix design been established by the Tollway Materials prior to construction?	____

# 3.	PREPARATION OF SUBGRADE (Article 302.06)

Prior to application of the modifier, is the subgrade shaped to the grades and cross sections shown on the plans?	____

Are the cut sections and/or at grade sections prepared according to Articles 301.03 and 301.04?	____

# 4.	APPLICATION (Article 302.07)

Is the modifier being distributed uniformly over the surface and limited to the area which can be incorporated in the soil during the same working day?	____

# 5.	MIXING (Article 302.08)

Is the modifier, soil, and water (if necessary) thoroughly mixed until a homogeneous layer is produced with 75% of the mixture smaller than
1 inch?	____

The use of fly ash may require more than one pass of a rotary speed mixer.  If more than one pass is required, is 25% of the fly ash applied and mixed with the first pass and the remainder applied and mixed with the second pass?	____

# 6.	COMPACTION (Article 302.09)

Depending on the type of modifier used, is compaction beginning as follows?	____

(1)	Portland Cement, Slag Modified Portland Cement, Fly Ash:  No later than 1 hour after mixing begins.

(2)	Hydrated Lime, Non-Hydrated Lime:  Within the same day as mixing.

(3)	Lime Slurry:  Within 24 hours of mixing.

(4)	By-Product Hydrated Lime:  Between 24 hours to 3 days after mixing.

Is the soil compacted to a dry density of not less than 95 percent of the laboratory standard dry density?	____

# 7.	FINISHING AND CURING (Article 302.10)

Is the final surface shaped to the required lines, grades and cross section shown on the plans?	____

If HMA base course, pavement (full-depth), Portland cement concrete base course, or pavement is to be constructed, is the surface of the modified soil brought to true shape and correct elevation according to Article 301.07?	____

Is the modified soil allowed to cure for 24 hours?	____